12. Building services engineering (BSE) systems

# Worksheet 13: Electrotechnical components (learner)

- Match the cable type to its use:

steel wired armoured, mineral insulated copper, FP200, PVC insulated, flexible, data multi-cored

| Cable type | Use |
| --- | --- |
|  | Traditionally used for the installation of fire protection systems but also used in large boiler house control systems because of their high resistance to fire and heat. |
|  | Designed to carry low voltages (data signals) from sensors to central hubs. |
|  | Suitable for electricity distribution underground and within buildings from switchgear to distribution boards. |
|  | The conductors are either multi-stranded or solid core. These are the preferred cables for external hard wiring. |
|  | A cable that has superseded MICC cable for firefighting alarm systems because it is easier to install and simpler to terminate. |
|  | Used mainly to connect appliances to hard wired power outlets or fused spurs. |

- Name three types of cable containment.