12. Building services engineering (BSE) systems

# Worksheet 8: Protection systems (tutor)

- Name four types of protection system.

Surveillance cameras, access control, fire protection, intruder alarm.

- Describe how heat sensors and smoke detectors work.

| Heat sensor | Smoke detector |
| --- | --- |
| When a heat sensor is subjected to heat generated by a fire, a bimetal strip inside the sensor bends causing two contacts to meet and allowing current to flow in the alarm circuit, sounding the fire alarm. | A smoke detector works by detecting smoke, not heat. A light source in the sensor shines on a photocell. If the light beam is deflected by smoke (ie no longer shines on the photocell), the alarm is triggered. |

- Name three types of access control system.

Card reader, fob sensor, keypad with access code.

- Label the following image of a fire protection system, identifying each of the components.