12. Building services engineering (BSE) systems

# Worksheet 9: Refrigeration systems (tutor)

- List four required properties of a refrigerant.

Favourable thermodynamic properties.

Noncorrosive to mechanical components.

Safe, including freedom from toxicity and flammability.

Does not cause ozone depletion or climate change.

- Fill in the blanks with either high or low pressure to explain how a compression cycle works.

The compression cycle is a system of four quadrants. 1) gas/liquid at low pressure; 2) gas at low pressure; 3) gas at high pressure; and 4) liquid at high pressure.

- Add these labels to this diagram of the vapour compression cycle:

compressor, condenser, evaporator, expansion device