# 2. Construction science principles

# Worksheet 13: Theory test (tutor)

1. Calculate the resistance of an electric kettle element if the supply voltage is 230V and the current flowing is 10A.

R = V

I

R = 230

10

R = 23Ω

2. Calculate the electrical current flowing in a circuit if the supply voltage is 230V and the resistance of its element is 30Ω.

I = V

R

I = 230

30

I = 7.67A

3. Calculate the supply voltage of a heating element if it has a current of 1A flowing through it and a resistance of 12Ω.

V = I x R

V = 1 x 12

V = 12V

4. Calculate the electrical current required to heat up a 230V kettle if its power rating is 2kW.

I = P

V

= 2000

230

= 8.69A