2. Construction science principles

# Worksheet 5: Researching mechanical principles (tutor)

A number of different terms are used for descriptions and explanations of mechanical principles.

Research the following terms used in mechanical principles and define each in a short sentence of no more than 20 words.

| Term | Definition |
| --- | --- |
| Energy | Energy is expressed as the amount of work done by a force. Energy cannot be created or destroyed, only converted from one form to another. The units of energy are joules (J). 1 joule is 1 newton (N) applied over a distance of 1 metre (m) (1J = 1Nm). |
| Work | Work done is the force x distance moved and is expressed in joules (J); 1 joule of work is done when a force of 1 newton causes a movement of 1 metre. This means that work done can be measured in newton metres (Nm) as well as in joules. |
| Power | Power is a measure of the rate of energy which is delivered to a process. Power = the amount of energy transferred per second, measured in watts (W). |