2. Construction science principles

# Worksheet 7: Moments and reactions (tutor)

Basic mechanical principles include the following:

- the theory of moments

- reactions.

Reaction left

Force

Reaction right

In order to answer the following questions, you will need to know some basic principles.

- Forces down must equal reactions up.

- A force or system of forces may cause an object to turn. A moment is the turning effect of a force. Moments act about a point in a clockwise or anticlockwise direction. Anti-clockwise moments = clockwise moments. For more explanation, see Moments - Moments, levers and gears - AQA - GCSE Physics (Single Science) Revision - AQA - BBC Bitesize.

- Bending moment = force x distance.

Force (F) is measured in newtons (N) or kilo Newtons (kN)

Distance (d) is measured in metres (m)

1. If the force on the beam is 200kN and the distance between supports is 6m, what is the value of the reactions?

Reaction left x 6m = 200kN x 3m

RL = 200 x 3/6 = 100kN

As the beam is symmetrical, the right reaction will be the same value.

2. Calculate the maximum bending moment at the centre of the beam.

At centre 200 x 3 - 100 x 3 = 300KN/m.