6. Construction measurement principles

# Worksheet A: Additional calculations (tutor)

10m

4m

8m

5m

The drawing below shows the foundations for a new house. The foundations are 1m wide and 1.5m deep.

Area 5

Area 4

Area 1

Area 2

Area 3

Area 6

- Calculate the volume of concrete required for the foundations.

Area 1: 5m x 1m x 1.5m = 7.5m3

Area 2: 5m x 1m x 1.5m = 7.5m3

Area 3: 4m x 1m x 1.5m = 6m3

Area 4: 2m x 1m x 1.5m = 3m3

Area 5: 8m x 1m x 1.5m = 12m3

Area 6: 8m x 1m x 1.5m = 12m3

Total = 48m3

- The ratio for the concrete for foundations is a mix of 1 part cement to 3 parts sand to 6 parts aggregates. Calculate:

(1 part + 3 parts + 6 parts = 10 parts total)

- How much cement is required?

48m3 / 10 parts = 4.8m3 cement

- How much sand is required?

48m3 /10 parts = 4.8m3 	4.8m3 x 3 parts = 14.4m3 sand

- How much aggregate is required?

48m3 /10 parts = 4.8m3 	4.8m3 x 6 parts = 28.8m3 sand