7. Building technology principles

# Worksheet 1: Onsite construction methods chart (tutor)

| Method | Construction context | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Timber frame construction | Used in domestic and commercial contexts primarily | - Timber is a sustainable building material - Can be erected faster than traditional methods - Offsite fabrication can improve quality | - Risks associated with moisture content and rot - Risk of fire |
| Steel frame system(s) (SFS) | Common method used in primarily commercial and industrial contexts but adoption in a residential context is rising | - Steel has a high strength to weight ratio - Offsite fabrication can improve quality - Units can be mass produced - Can be clad with a variety of materials | - Buckling can be an issue with steel structures - High maintenance costs - Fireproof treatment - Health and safety concerns when welding activities take place onsite |
| Straw bale construction | Popular for self-build and community projects. Can be used in residential or commercial contexts | - Straw is a renewable material - Good thermal insulation properties - Lightweight, so reduces loads on the foundations - Requires minimal skills for installation so can be suited to self-build and community-based projects | - Straw has low compressive strength - Inconsistent material properties - Limited resilience to moisture - Use is limited to above damp proof course (DPC) level |
| Brick and block construction | Most common form of construction for domestic dwellings | - Can withstand large amounts of compressive weight - Materials are readily accessible and available - Bricks have excellent thermal mass - Bricks and blocks can be recycled | - Time consuming in comparison to other more modern construction methods - Weather conditions can impact this form of construction - Materials need to be stored onsite and be well protected from the elements |
| Container construction | Initially used for small-scale residential projects but increasing in popularity for commercial use such as offices, classrooms and self-storage units | - Design versatility - Can be cost-effective in terms of initially purchasing the containers - Durability | - Insulation requirements can be complex to achieve - Meeting building regulations with this construction method can be challenging - Transportation to site and lifting containers into the site boundary may need to be considered and planned efficiently - Sourcing skilled labour with experience in container construction |
| Portal frame | Predominantly used in agricultural, industrial and commercial contexts | - Simple structural from - Creates a large, clear span - Can be made weatherproof at a low cost - Can be custom made to meet design and client requirements | - Span lengths can be restricted depending on the prominent building material |
| Robotics | Large scale residential, commercial or industrial projects | - Can help address low productivity - Can help reduce accidents and injuries - Reduces human error | - Staff upskilling and training - Technical issues will need to be resolved by specialists - High initial investment costs |