7. Building technology principles

# Worksheet 22: Building structure and fabric (tutor)

- Provide a definition for the term building fabric .

The external fabric consists of the materials and components that form the shell of a building. This includes walls, floors, roofs, windows and doors.

- What does the building fabric do? Think of four things.

Protects occupants from the elements

Regulates the internal environment in terms of temperature, humidity and moisture

Provides privacy and security for occupants

Provides access from the interior to the exterior of the building

- State three factors to be considered when selecting building materials

Building regulation requirements/structural properties of the materials

Location of use (internal/external)

Environmental factors

Cost implications

- Briefly explain the possible consequences of selecting unsuitable fixtures and supports.

Falling objects

Reduced lifespan of equipment

- Describe three uses of cladding.

Retrofit improvement

To conceal the primary structure

To improve thermal efficiency

To provide aesthetic value

To reduce noise transmission

Buildability

- State two advantages of steel as a building material.

High strength to weight ratio

Relatively low cost

Versatility

Reliability

- Briefly outline the materials that can be used in masonry construction .

Mortar (cement, additives, water, lime etc)

Bricks (engineering bricks, facing bricks, etc.)

Blocks (solid concrete, hollow concrete blocks, etc)

- Explain what is meant by the term fenestration.

Refers to openings in the building fabric in relation to the quality, quantity and characteristics of the building’s aesthetics. Openings include windows, doors and skylights.