7. Building technology principles

# Worksheet 6: Foundation types (tutor)

Fill in the chart to show in what context each foundation type can be used and the possible advantages and disadvantages of each type.

| Foundation type | Uses | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Narrow strip | For low-rise domestic developments up to three storeys such as houses or extensions etc | Installation costs Easy to install/‘form’ (a commonly used term in the industry) Ability to withstand loads | Not suitable on soil with poor load-bearing capacity Trench support needed |
| Wide strip | For heavy structures where soil is soft or has poor load-bearing capacity | Easy to install/form | Trench support needed |
| Raft | Used on poor soils for lightly loaded buildings where subsidence may be expected | Provides a good foundation where soil conditions are variable | Formwork required Costly compared to other options |
| Pad | For use in commercial contexts such as portal frame structures | Little excavation required Reinforcement for tension and shear can be incorporated | Foundation size can be very large to cope with high point loads Limited use – predominantly for framed structures |