3. Engineering representations

Worksheet 14: Limits and fits (learner)

Using the tables provided on the following pages, calculate the upper and lower limit of both the shaft and the bore. Can you infer the type of fit from your calculations?

|  | Bore size | Shaft size |
| --- | --- | --- |
| Type of fit = |  |  |
| Type of fit = |  |  |
| Type of fit = |  |  |

Table 1: Shaft fits

| Type of fit |  | Clearance |  |  |  |  |  | Transition |  | Interference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shaft tolerance |  | c11 | d10 | e9 | f7 | g6 | h6 | k6 | n6 | p6 | s6 |
| Hole tolerances | H7 |  |  |  |  |  |  |  |  |  |  |
|  | H8 |  |  |  |  |  |  |  |  |  |  |
|  | H9 |  |  |  |  |  |  |  |  |  |  |
|  | H11 |  |  |  |  |  |  |  |  |  |  |

Table 2: Tolerance limits for bores/holes

| Basic sizes |  | Tolerance unit = 0.001mm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Over | Up to and incl. | H7 |  | H8 |  | H9 |  | H11 |  |
|  |  | ES+ | EI | ES+ | EI | ES+ | EI | ES+ | EI |
| mm 3 | mm 6 | 12 | 0 | 18 | 0 | 30 | 0 | 75 | 0 |
| 6 | 10 | 15 | 0 | 22 | 0 | 36 | 0 | 90 | 0 |
| 10 | 18 | 18 | 0 | 27 | 0 | 43 | 0 | 110 | 0 |
| 18 | 30 | 21 | 0 | 33 | 0 | 52 | 0 | 130 | 0 |
| 30 | 50 | 25 | 0 | 39 | 0 | 62 | 0 | 160 | 0 |
| 50 | 80 | 30 | 0 | 46 | 0 | 74 | 0 | 190 | 0 |
| 80 | 120 | 35 | 0 | 54 | 0 | 87 | 0 | 220 | 0 |
| 120 | 180 | 40 | 0 | 63 | 0 | 100 | 0 | 250 | 0 |
| 180 | 250 | 46 | 0 | 72 | 0 | 115 | 0 | 290 | 0 |

Table 3: Tolerance limits for shafts

| Basic sizes |  | Tolerance unit = 0.001mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Over | Up to and incl. | c11 |  | d10 |  | e9 |  | f7 |  | g6 |  | h6 |  | k6 |  | n6 |  | p6 |  | s6 |  |
|  |  | es – | ei – | es – | ei – | es – | ei – | es – | ei – | es – | ei – | es – | ei – | es + | ei + | es + | ei + | es + | ei + | es + | ei + |
| mm 3 | mm 6 | 70 | 145 | 30 | 78 | 20 | 50 | 10 | 22 | 4 | 12 | 0 | 8 | 9 | 1 | 16 | 8 | 20 | 12 | 27 | 19 |
| 6 | 10 | 80 | 170 | 40 | 98 | 25 | 61 | 13 | 28 | 5 | 14 | 0 | 9 | 10 | 1 | 19 | 10 | 24 | 15 | 32 | 33 |
| 10 | 18 | 95 | 205 | 50 | 120 | 32 | 75 | 16 | 34 | 6 | 17 | 0 | 11 | 12 | 1 | 23 | 12 | 29 | 18 | 39 | 28 |
| 18 | 30 | 110 | 240 | 65 | 149 | 40 | 92 | 20 | 41 | 7 | 20 | 0 | 13 | 15 | 2 | 28 | 15 | 35 | 22 | 48 | 35 |
| 30 | 40 | 120 | 280 | 80 | 180 | 50 | 112 | 25 | 50 | 9 | 25 | 0 | 16 | 18 | 2 | 33 | 17 | 42 | 26 | 59 | 43 |
| 40 | 50 | 130 | 290 | 80 | 180 | 50 | 112 | 25 | 50 | 9 | 25 | 0 | 16 | 18 | 2 | 33 | 17 | 42 | 26 | 59 | 43 |
| 50 | 65 | 140 | 330 | 100 | 220 | 60 | 134 | 30 | 60 | 10 | 29 | 0 | 19 | 21 | 2 | 39 | 20 | 51 | 32 | 72 | 53 |
| 65 | 80 | 150 | 340 | 100 | 220 | 60 | 134 | 30 | 60 | 10 | 29 | 0 | 19 | 21 | 2 | 39 | 20 | 51 | 32 | 78 | 59 |
| 80 | 100 | 170 | 390 | 120 | 260 | 72 | 159 | 36 | 71 | 12 | 34 | 0 | 22 | 25 | 3 | 45 | 23 | 59 | 37 | 93 | 71 |
| 100 | 120 | 180 | 400 | 120 | 260 | 72 | 159 | 36 | 71 | 12 | 34 | 0 | 22 | 25 | 3 | 45 | 23 | 59 | 37 | 101 | 79 |
| 120 | 140 | 200 | 450 | 145 | 305 | 85 | 185 | 43 | 83 | 14 | 39 | 0 | 25 | 28 | 3 | 52 | 27 | 68 | 43 | 117 | 92 |
| 140 | 160 | 210 | 460 | 145 | 305 | 85 | 185 | 43 | 83 | 14 | 39 | 0 | 25 | 28 | 3 | 52 | 27 | 68 | 43 | 125 | 100 |
| 160 | 180 | 230 | 480 | 145 | 305 | 85 | 185 | 43 | 83 | 14 | 39 | 0 | 25 | 28 | 3 | 52 | 27 | 68 | 43 | 133 | 108 |
| 180 | 200 | 240 | 530 | 170 | 355 | 100 | 215 | 50 | 96 | 15 | 44 | 0 | 29 | 33 | 4 | 60 | 31 | 79 | 50 | 151 | 122 |
| 200 | 225 | 260 | 550 | 170 | 355 | 100 | 215 | 50 | 96 | 15 | 44 | 0 | 29 | 33 | 4 | 60 | 31 | 79 | 50 | 159 | 130 |
| 225 | 250 | 280 | 570 | 170 | 355 | 100 | 215 | 50 | 96 | 15 | 44 | 0 | 29 | 33 | 4 | 60 | 31 | 79 | 50 | 169 | 140 |