3. Engineering representations

# Worksheet 6: Drawing types (tutor)

Complete the table below.

| Type of drawing | Description | Used by? |
| --- | --- | --- |
| Freehand sketch | Drawn without measurements using a pencil and eraser, a quick drawing used to present an idea eg sketch of a car design idea | Shop floor staff – quick, on the job sketches where computers are not available. Or where a pencil sketch is simply quicker, to be redrawn later Designers to illustrate an idea to others |
| Orthographic projection | Combines individual drawings to depict how the same component will look when viewed from a different angle eg single component drawings | Gives a craftsperson enough information to be able to repair, install or create the item |
| Isometric | Used to show a 2D drawing in a 3D view. Generally used to illustrate what the end product will look like eg house design, complex components | Anyone who needs a good idea of what the finished article will look like – technicians, architects, craftspeople |
| Assembly | Illustrations used to explain how parts fit together in a larger assembly eg how to assemble a complex part of a structure, machinery etc | Maintenance/repair staff, installation staff |
| Exploded | Shows how the various parts go together to form the finished design/object eg instructions for flat-pack furniture use exploded views | Installation, maintenance and repair staff (for ordering spare parts) |
| Detail | Contains the key points to enable the manufacture or description of a single component eg drawing of a piston | Anyone wanting to manufacture a product where more detailed views are needed. Usually used for highly complex parts combined with a sectional view |
| Block diagram | Provides a high-level, functional view of a system in a graphical representation Contains ‘blocks’ that show the relationships between the stages/items/people in a system eg diagram showing the process of a water treatment plant | Manufacturing engineers wanting to check or follow a process |
| Flow chart | A diagram that illustrates a process, system or computer algorithm Widely used to document, study, plan, improve and communicate complex processes in clear, easy-to-understand diagram eg chart to show a manufacturing process step-by-step | Anyone seeking to understand the order of a process |
| Circuit diagram | A graphical representation of an electrical circuit used to show the components and wiring eg diagram of a lighting circuit in a domestic house | Electricians |
| Schematic | Symbols are used to simplify a complicated system which would be too complex to show by drawing the components eg schematic of an aviation hydraulic system | Anyone wanting to install the system |