3. Engineering representations

# Worksheet 7: Key features of an engineering drawing (tutor)

Highlight and identify all the key features on the drawing.

These include symbols and line types.

Students should be assisted by the tutor in this task as required.

| Feature | Meaning |
| --- | --- |
|  | Surface to be finished using milling operations |
|  | Illustrates the centre point of a radius |
|  | The maximum and minimum size the measurement can be |
|  | Surface finish must be machined to a finish of 12.5 microns |
|  | There are three holes with a diameter of 16mm |
|  | A dimension in brackets is an accumulative dimension that is not subject to any tolerances. The individual dimensions that make up the total have the tolerances applied to them instead |
|  | A dashed line represents hidden detail on a drawing: items that are obscured from view but still need to be illustrated |
|  | R is the designation for the size of a radius on an item |
|  | Centre lines show the centre of a round, cylinder-shaped object. Usually but not restricted to holes. They must extend beyond the centre of the object. Dimension can be added |
|  | Symbol used to show which orthographic projection has been used. In this case, the drawing is third (3rd) angle |
|  | The title block on an engineering drawing contains critical information about the drawing including the author, name, tolerances etc |