4. Essential mathematics for engineering and manufacturing

# Worksheet 13.2: Volumes of 3D shapes: contextual (tutor)

- The tank shown below is filled with water until it is 75% full.

How many litres of water are in the tank?

1 litre=0.001 m3

2.5×0.5×1.5=1.875 m3

0.75×1.875=1.40625 m3

1 litre=0.001 m3

1.40625÷0.001=1406.25 litres

- Aluminium is delivered in cuboid blocks, as shown in the diagram below (A).

It is then melted and poured into a hollow cylindrical mould of uniform thickness to make a tube with dimensions as shown (B).

How much aluminium is left over when the cylindrical mould is full?

It is assumed that the heating and cooling process does not change the initial volume of the aluminium.

Volume of block=125×60×100=750 000 mm3

Volume of large cylinder=252×π×1000=625 000π mm2

Volume of small cylinder=202×π×1000=400 000π mm2

Volume of tube=625000π-400000π=225 000π mm3

Aluminium remaining after mould=750 000-225 000π=43 141.653 mm3

43 100 mm3=43.1 cm3

- A swimming pool is designed to have a shallow end and a deep end with a gradual slope between them.

The pool is 10 m wide.

The length of the pool is 25 m

The shallow end has a 1 m depth.

The deep end has a depth of 2.5 m.

Below is a diagram of the cross section of the pool, but the measurements have been omitted.

- Using the information above, label the diagram.

- Hence determine the volume of the swimming pool in cubic metres.

25 m

1 m

2.5 m

Cross sectional area=25×(2.5+1)2=43.75 m2

Volume=43.75×10=437.5 m3

- A generic bolt cast is made by filling the mould shown. Calculate the volume of steel, in m3, needed produce 1000 screws.

Volume=π×2.52×2+π×1.52×8=612π mm3

Volume of 1000 screws=1000×61π2=30 500π mm3

30 500π=95 800 mm3 3sf

95 800÷10003=9.58×10-5m3