4. Essential mathematics for engineering and manufacturing

# Multiple choice questions (tutor)

Find 316of 1539.

- Give your answer correct to 3 significant figures.

- 288

- 288.563

- 289

- 290

A TV screen is manufactured using the ratio of width to height 16:9. It is then advertised using the diagonal length the size of the tv.

- Calculate the length and width of a TV screen (to the nearest inch) that is advertised at 55 inches.

- 16 inches by 9 inches

- 48 inches by 27 inches

- 24 inches by 13.5 inches

- 24 inches by 14 inches

3.	Solve 3x+2=4x-3

a.	79

b.	73

c.	3

d.	9

4.	The formula for kinetic energy is EK=12mv2

Rearrange the equation to make v the subject.

a.	v=12EKm2

b.	v2=2EKm

c.	v=2 EKm

d.	v=EK 2m

5.	Determine the solutions to the equation 2x2+5x=12

a.	x=1.5  and x=-4

b.	x=-1.5  and x=-4

c.	x=1.5  and x=4

d.	x=-1.5  and x=4

6.	Simplify 3x7y62

a.	3x9y8

b.	9x9y8

c.	3x14y12

d.	9x14y12

7.	Find the missing angle x in the triangle below. Give your answer to the nearest degree.

80°

12 m

x°

25 m

a.	45°

b.	28°

c.	41°

d.	60°

8.	Use logarithms to solve 42x-1=95x-5 . Give your answer correct to 2 decimal places.

a.	1.00

b.	1.16

c.	1.17

d.	1.87

9.	A tank can hold 360 litres of water when full.

1 litre of water is equivalent to 1000 cubic centimetres.

Given that the width of the tank is 1.2 m and the depth is 0.75 m, determine the height of the tank.

a.	0.4 m

b.	0.45 m

c.	0.5 m

d.	0.55 m

10.	A chemical engineering company is testing a new product to see how the chemical dissolves material over time. Measurements are taken at intervals as shown in the table below.

| Time (t) in hours. | 3 | 5 | 8 | 10 | 15 |
| --- | --- | --- | --- | --- | --- |
| Surface dissolved ( s ) in cm2. | 16.3 | 33.3 | 64.3 | 87.9 | 155.1 |

The data is found to follow a trend of s=atn where a and n are constants.

Use logarithms to estimate the values of a and n correct to 1 decimal place.

a.	a = 16.1 and n = 3.5

b.	a = 1.4 and n = 3.5

c.	a = 3.5 and n = 16

d.	a = 3.5 and n = 1.4

11.	Calculate the area of the compound shape. Give your answer correct to the nearest whole number.

50°

12 cm

a.	251 cm2

b.	246 cm2

c.	166 cm2

d.	129 cm2

12.	Given that

A=3527 and B=4a-37

Determine the value of 3A – B.

a.	Cannot be done.

b.	515-a914

c.	515-a314

d.	5a-15314

13.	Evaluate 05x+2x-1x-5 dx

Give your answer as an exact fraction.

a.	57512

b.	-57512

c.	23544

d.	-2354

14.	A firework is projected from a horizonal plane and follows a path that can be modelled by the equation h=42t-6t2. Where h is the height in m above the ground and t is the time in seconds.

Use differentiation to find the maximum height of the firework.

a.	3.5 m

b.	7 m

c.	42 m

d.	73.5 m

15.	Calculate the 50th term of the following sequence

65   	97.5   	146.25   	219.375

Give your answer in standard form correct to 2 decimal places.

a.	4.14 ×1010

b.	2.76 ×1010

c.	1.69 ×103

d.	1.66 ×103

16.	The area of the sector is 256π cm2.

Given that the angle in the sector is 60°, calculate the radius of the sector.

a.	5 cm

60°

b.	5.2 cm

c.	5.8 cm

d.	6.1 cm

17.	A car is being tested and its displacement from the starting position is given by the equation s=23t3-4t2+80t-14

where s is the displacement of the car in metres and t is the time in seconds.

Find the maximum velocity of the car.

a.	2 ms-1

b.	14 ms-1

c.	72 ms-1

d.	135 ms-1

18.	Convert the binary number 1001110001000 to a hex number.

a.	1388

b.	148F

c.	22AE

d.	3300

19.	A builders’ merchants sells bags of aggregate advertised at a weight of 830 kg for a bulk bag.

A quality assurance test is carried out to ensure that the bags are being weighed correctly,

A sample of 8 bags is taken and weighed to the nearest gram.

Calculate the standard deviation of the sample.

| 830 150 | 830 200 | 829 755 | 830 005 |
| --- | --- | --- | --- |
| 829 650 | 829 114 | 830 110 | 830 045 |

a.	340.28

b.	363.77

c.	829878.625

d.	6639029

20.	A sample of 50 bricks is taken and tested for defects in absorption and hardness.

3 bricks fail both the absorption and hardness test.

7 bricks fail the absorption test but not the hardness test.

7 bricks fail the hardness test.

What percentage of bricks pass both tests?

a.	72%

b.	36%

c.	28%

d.	14%