5. Essential science for engineering and manufacturing

# Worksheet 14: Thermal expansion (learner)

- A steel bar is 15 m long at a temperature of 12 °C. Calculate the change in length when its temperature is increased to 240 °C.

The value of the coefficient of linear expansion for the steel is 16 × 10−6 K−1.

- A sheet of material is 7500 m long at 20 °C. Its length increases by 0.6375 m at 70 °C.

Calculate the coefficient of expansion of the material.

- A brass component is 500 mm long at 20 °C. Determine the temperature at which the length of the component has increased to 505 mm due to thermal expansion.

Take the coefficient of linear expansion of brass to be 20 × 10−6 °C−1.

- An aluminium bar is 200 mm long at 15 °C and is heated to 200 °C.

Calculate the length of the bar at 200 °C.

Take the coefficient of linear expansion of aluminium to be 23 × 10−6 °C−1.