5. Essential science for engineering and manufacturing

# Worksheet 14: Thermal expansion (tutor)

- A steel bar is 15 m long at a temperature of 12 °C. Calculate the change in length when its temperature is increased to 240 °C.

The value of the coefficient of linear expansion for the steel is 16 × 10−6 K−1.

Answer: 	Increase in length =  × L × T = 16 × 10−6 × 15 000 × 228 = 54.72 mm

- A sheet of material is 7500 m long at 20 °C. Its length increases by 0.6375 m at 70 °C.

Calculate the coefficient of expansion of the material.

Answer: 	L2 = L1 [1 + α (T2 − T1)]

7500.6375 = 7500 [1 + α(70 − 20)] = 7500 + 7500 × α × 50

Rearranging gives α = 7500.6375 - 75007500 × 50 = 1.7 × 10−6 K−1

- A brass component is 500 mm long at 20 °C. Determine the temperature at which the length of the component has increased to 505 mm due to thermal expansion.

Take the coefficient of linear expansion of brass to be 20 × 10−6 °C−1.

Answer: 	Increase in length =  × L × T

Rearranging ∆T=∆Lα × L=505 - 50020 × 10-6 × 500 = 500 °C

Temperature = 20 °C + 500 °C = 520 °C

- An aluminium bar is 200 mm long at 15 °C and is heated to 200 °C.

Calculate the length of the bar at 200 °C.

Take the coefficient of linear expansion of aluminium to be 23 × 10−6 °C−1.

Answer: 	Increase in length =  × L × T = 23 × 10−6 × 200 × 185 = 0.851 mm

Length at 200 °C = original length + increase = 200 + 0.851 = 200.851 mm