6. Materials and their properties

# Worksheet 11: Galvanic protection (tutor)Top of Form

- Bottom of Form

- Briefly explain the process of galvanising.

Galvanising involves the application of a zinc coating to the surface of a metal. It is primarily a protective measure used to prevent corrosion, hence increasing the life of the protected component. Two types of protection are achieved: barrier and galvanic.

- Conduct some research into common uses of galvanic protection. Make a list here.

Learners will have identified numerous uses for this protection process, including:

- pipelines

- water tanks

- marine (ships, harbours, offshore structures).

- Outline the basic principle of barrier protection.

When a zinc coating is applied to a surface, it dries and hardens to form a protective zinc oxide layer or barrier that is impervious to air and moisture, eliminating one of the components needed for corrosion to occur – the electrolyte. Because the underlying steel substrate is unable to come into contact with external air or moisture, corrosion is prevented.

- What is meant by preferential corrosion?

Galvanic protection offers preferential corrosion, meaning that the zinc coating will corrode first if the object is exposed to external air or moisture. This protection will continue until the coating is entirely consumed (corroded away).