6. Materials and their properties

# Worksheet 14: Material testing methods (tutor)Top of Form

- Bottom of Form

While two testing methods have been suggested, any number can be chosen by the tutor, depending on time available and context. Some or all could be set as homework or as part of an ongoing project.

Research two of the testing methods identified below and outline the following in the form of a short report or presentation.

- Name of test method

- Type of test (destructive or non-destructive)

- Material properties being tested

- Equipment required

- An overview of the equipment set-up (sketch or image)

- Sample shape and size

- Process of test

- Results and collection methodology

- Safety considerations

Testing methods

- Tensile test

- Charpy impact test

- Fatigue test (Wohler test)

- Brinell hardness test

Tensile test

- Name of test method

Tensile test

- Type of test (destructive or non-destructive)

Destructive test as the sample is destroyed

- Material properties being tested

Tensile strength

Elasticity

Yield strength

- Equipment required

Tensile testing machine

- An overview of the equipment set-up

- Sample shape and size

- Process of test

  - Cut or mould specimen into dumbbell shapes.

  - Load the specimen into tensile grips.

  - Attach the extensometer to the sample.

  - Begin the test by separating the tensile grips at a constant rate of speed. Speed depends on specimen shape and can range from 1– 5mm.

  - The target time from start of test to break should be from 30 seconds to 5 minutes.

  - End the test after sample break (rupture).

- Results and collection methodology

Data is collected by the machine and downloaded into a load extension plotted graph. The raw data is also available in a tabulated form for reference.

- Safety considerations

Protective PPE should be worn at all times during the testing process and the equipment should only be operated by competent persons.

Charpy impact test

- Name of test method

Charpy impact test

- Type of test (destructive or non-destructive)

Destructive test as the sample is damaged

- Material properties being tested

Toughness

- Equipment required

Charpy V-notch tester

- An overview of the equipment set-up

- Sample shape and size

- Process of test

- The Charpy impact test entails striking a notched impact specimen with a swinging weight attached to a swinging pendulum.

- The specimen breaks at its notched cross-section upon impact, and the upward swing of the pendulum is used to determine the amount of energy absorbed (notch toughness) in the process.

- Energy absorption is directly related to the brittleness of the material.

- Since temperature can affect the toughness of a material, the Charpy test is performed at a series of temperatures to show the relationship of ductile to brittle transition in absorbed energy.

- Results and collection methodology

Data is collected by the machine and downloaded into the raw data available in a tabulated form for reference.

- Safety considerations

Protective PPE should be worn at all times during the testing process and the equipment should only be operated by competent persons.

Fatigue test (Wohler test)

- Name of test method

Fatigue test (Wohler test)

- Type of test (destructive or non-destructive)

Destructive test as the sample is destroyed

- Material properties being tested

Fatigue

Brittleness

Corrosion resistance

- Equipment required

Fatigue tester

- An overview of the equipment set-up

- Sample shape and size

Cylinder or bar depending on material under test

- Process of test

  - To perform a fatigue test ,a sample is loaded into a fatigue tester or fatigue test machine and using a predetermined test stress.

  - It is then unloaded to either zero load or an opposite load.

  - This cycle of loading and unloading is then repeated until the end of the test is reached.

  - The test may be run to a predetermined number of cycles or until the sample has failed, depending on the parameters of the test.

- Results and collection methodology

Data is collected by the machine and downloaded into the raw data available in a tabulated form for reference.

- Safety considerations

Protective PPE should be worn at all times during the testing process and the equipment should only be operated by competent persons.

Brinell hardness test

- Name of test method

Brinell test

- Type of test (destructive or non-destructive)

Destructive test as the sample is damaged during the testing procedure

- Material properties being tested

Hardness

- Equipment required

Brinell hardness testing machine

Indenter sphere

Brinell microscope (to measure the generated impression)

- An overview of the equipment set-up

- Sample shape and size

Tungsten carbide ball

- Process of test

  - The Brinell hardness test is performed in a Brinell hardness test unit.

  - In this test method, a predetermined force (F) is applied to a tungsten carbide ball of fixed diameter (D) and held for a predetermined time period.

  - The force is then removed.

  - The spherical indenter creates an impression (permanent deformation) on the test metal piece.

  - This indentation is measured across two or more diameters and then averaged to get the indentation diameter (d).

  - Using this indentation size (d) the Brinell hardness number (BHN) is found using a chart or calculated using the Brinell hardness test formula.

- Results and collection methodology

The BHN is found using a chart or calculated using the Brinell hardness test formula.

- Safety considerations

Protective PPE should be worn at all times during the testing process and the equipment should only be operated by competent persons.