6. Materials and their properties

# Worksheet 15: Destructive and non-destructive testing (tutor)

Complete the table below, individually or in small groups. Some research will be required in addition to what you have already learned. The first item has been completed for you.

|  | Destructive testing | Non-destructive testing |
| --- | --- | --- |
| Cost | Expensive to operate due to equipment needed | Cheaper to operate |
| Time | More inspection time required | Can be automated at very high speed |
| Purpose | To identify the physical and chemical properties of the test material | To find defects in the test material |
| Examples | Tensile testing, corrosion testing | Radiography, ultrasonic testing |
| Damage | Sample is destroyed during testing | Sample is retained after testing |
| Uses | Used during the design process | Used to identify early signs of deterioration |
| Benefits | Reliable and accurate data from testing | All items can be tested without consequence |
| Limitations | Always requires a competent operator. Data applies to the test specimen only | Usually requires a competent operator |