6. Materials and their properties

# Worksheet 16: Hooke’s Law and Young’s modulus (tutor)Top of Form

- A graph of force (F) versus elongation (x) is shown in the figure below. Find the spring constant.

Hooke’s law formula:			k = F/x

F = force (newton)

k = spring constant (newton/metre)

x = the change in length (metre)

Spring constant:

k = 10/0.02 = 20/0.04

k = 500 N/m

- Determine the spring constant from the data given.

| Number | Force (N) | Change in length (m) |
| --- | --- | --- |
| 1 | 5 | 0.01 |
| 2 | 10 | 0.02 |
| 3 | 15 | 0.03 |
| 4 | 20 | 0.04 |

Spring constant:		k 	=	 F/x

K1 	= 	5/0.01 		=	500N/m

K2 	= 	10/0.02 	=	500N/m

K3 	= 	15/0.03 	=	500N/m

K4 	= 	20/0.04	=	500N/m

k	= 	500N/m

- A wire of length 2m and a cross-sectional area of 10-4m2 is stretched by a load of 102kg by a distance of 0.1cm. Calculate the longitudinal stress, strain and Young’s modulus.

Initial length of wire (L) 	= 	2m

Cross-section area (A)	=	10-4m

Stretching weight 		=	102 Kg	=		102 x 9.8Nl

Increase in length (l)		=	0.1cm 		=	0.1 x 10-2m

Therefore:

Stress 	=	F/A

=	mg/A

= 	(102 x 9.8)/10-4

=	1 x 107 N/m2

Strain		=	1/L

=	1 x 10-3/2

=	0.5 x 10-3

= 	5 x 10-4

Young’s modulus (Y)

Y		=	Stress/strain

=	1 x 107/ 5 x 10-4

=	2 x 1010 N/m2