6. Materials and their properties

# Worksheet 17: Interpreting results (learner)Top of Form

- The following data have been obtained from conducting a tensile test on a material specimen of diameter 10mm and original length 60mm.

| Load (kN) | 16 | 32 | 56 | 72 | 95 | 110 | 132 | 142 | 140 | 135 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extension (mm) | 0.2 | 0.4 | 0.7 | 0.9 | 1.5 | 2.5 | 5.0 | 8.5 | 10.0 | 12.0 |

Using graph paper or a suitable computer package, plot a load–extension graph and determine the following for the sample material: tensile strength, 0.1% proof stress, Young’s modulus for the specimen.