6. Materials and their properties

# Worksheet 9: Effects of processing techniques (tutor)

- For each of the following processes, explain how the material properties will be affected, including reference, as appropriate, to the following properties. Include diagrams or illustrations of the process to aid your explanations.

  - Ductility

  - Hardness

  - Toughness

  - Brittleness

  - Corrosion resistance

Process 1

Rolling of steel ingots into sheet steel using both hot and cold rollers as part of the process.

The hot rolling process has a significant effect on the properties of steel. By heating steel above its recrystallisation temperature and then cooling it at room temperature in a process known as normalisation, the steel’s microstructure undergoes permanent changes, resulting in enhanced toughness and ductility. This additional ductility makes the metal easy to form and bend. The hot rolling process also affects the overall shape of the finished metal piece. Hot rolled forms can be much larger than cold rolled forms. However, since hot rolled steel is heated and cooled, it is susceptible to shrinkage, giving less control over the shape and size of the finished product.

Process 2

Injection moulding of thermoplastic (ABS) designed for use in the automotive industry.

ABS is an engineering-grade thermoplastic with a relatively low melting point that makes it easy to mould. This opaque polymer supports the use of colourants and is available with different textures and surface finishes. ABS is known for its strength and impact resistance. It is corrosion resistant, and as it is a non-metallic material it also retains a level of flexibility to absorb impact and thermal shock.