7. Mechanical principles

# Worksheet 10: Power and work (learner)

What are the units for power?

- Ohms

- Amps

- Watts

- Joules

- Which of the following is a definition of power?

- The amount of work done by a certain force

- The energy transferred × time taken

- The amount of work done ÷ time taken

- A certain force ÷ the amount of work done

- A machine uses 10 000J of energy in 4s. What is its power?

- 40 000W

- 2500W

- 25 000W

- 4000W

- State D’Alembert’s principle.

- Calculate the work done when a force of 40N pushes an object 500m in the same direction as the force.

- Calculate the work done when a mass is lifted vertically by a crane to a height of 5m, the force required to lift the mass being 98N.

- Water is pumped vertically upwards through 50m and the work done is 294.3kJ. Determine the number of litres of water pumped (1 litre of water has a mass of 1kg).

- Calculate the power required to lift a mass through a height of 10m in 20s if the force required is 3924N.