10. Engineering and manufacturing control systems

# Worksheet 6: Signals (tutor)

- Using notes and sketches, explain the differences between analogue and digital signals.

- Analogue signals transmit information as a continuous set of values.

- Analogue signals are shown as sinusoidal waves. For example, radio waves.

- Digital signals transmit information as a sequence of discrete values (eg 1s and 0s).

- Digital signals are finite.

- Digital signals are shown as square waves.

- Using notes and sketches, explain the differences between PWM and PAM.

- Pulse width modulation (PWM) is where the width of the pulsed carrier signal is varied according to the amplitude of the message signal.

- Pulse amplitude modulation (PAM) is where the amplitude of the pulsed carrier signal is changed according to the amplitude of the message signal.